Grasmere Village Precinct

PWS # 0911020   0911030

2026 Report (2025 Data)

|  | LEAD AND COPPER |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contaminant (Units) |  | Action Level (AL) | 90th percentile sample value | Range of tap sampling results | Date | # of sites above AL | Action Level Exceedance YES/NO | Likely Source of Contamination | Health Effects of Contaminant |  |
| Copper (ppm) |  | 1.3 | .061 |  | 11/2024 | 0 | No | Corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives | Copper is an essential nutrient, but some people who drink water containing copper in excess of the action level over a relatively short amount of time could experience gastrointestinal distress. Some people who drink water containing copper in excess of the action level over many years could suffer liver or kidney damage. People with Wilson’s Disease should consult their personal doctor. |  |
| Lead (ppb) |  | 15 | .004 |  | 11/2024 | 0 | No | Corrosion of household plumbing systems, erosion of natural deposits | (15 ppb in more than 5%) Infants and young children are typically more vulnerable to lead in drinking water than the general population. It is possible that lead levels at your home may be higher than at other homes in the community as a result of materials used in your home’s plumbing. If you are concerned about elevated lead levels in your home’s water, you may wish to have your water tested and flush your tap for 30 seconds to 2 minutes before using tap water. Additional information is available from the Safe Drinking Water Hotline (800-426-4791). (Above 15 ppb) Infants and children who drink water containing lead in excess of the action level could experience delays in their physical or mental development. Children could show slight deficits in attention span and learning abilities. Adults who drink this water over many years could develop kidney problems or high blood pressure. |  |

| DETECTED WATER QUALITY RESULTS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Microbiological Contaminants |  |  |  |  |  |  |  |  |  |
| Contaminant (Units) | Level Detected* | Date | MCL | MCLG |  | Violation YES/NO |  | Likely Source of Contamination | Health Effects of Contaminant |
| Turbidity (NTU) | .035 | 2025 | TT | N/A |  | No |  | Soil runoff | Turbidity has no health effects. However, turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |
| Contaminant (Units) | Level Detected* | Date | MCL | MCLG | Violation YES/NO |  | Likely Source of Contamination |  | Health Effects of Contaminant |
| Barium (ppm) | .0107 | 2025 | 2 | 2 | No |  | Discharge of drilling wastes; discharge from metal refineries; erosion of natural deposits |  | Some people who drink water containing barium in excess of the MCL over many years could experience an increase in their blood pressure. |
| Contaminant (Units) | Level Detected* | Date | MCL | MCLG | Violation YES/NO |  | Likely Source of Contamination |  | Health Effects of Contaminant |
| Chlorine (ppm) | .30-1.6 | 2025 | MRDL= 4 | MRDLG= 4 | No |  | Water additive used to control microbes |  | Some people who use water containing chlorine well in excess of the MRDL could experience irritating effects to their eyes and nose. Some people who drink water containing chlorine well in excess of the MRDL could experience stomach discomfort. |
| Fluoride (ppm) | .69 | 2025 | 4.0 | 4.0 | No |  | Erosion of natural deposits; water additive which promotes strong teeth; discharge from fertilizer and aluminum factories |  | Some people who drink water containing fluoride in excess of the MCL over many years could get bone disease, including pain and tenderness of the bones. Fluoride in drinking water at half the MCL or more may cause mottling of children’s teeth, usually in children less than nine years old. Mottling, also known as dental fluorosis, may include brown staining and/or pitting of the teeth, and occurs only in developing teeth before they erupt from the gums. |
| Contaminant (Units) | Level Detected* | Date | MCL | MCLG | Violation YES/NO |  | Likely Source of Contamination |  | Health Effects of Contaminant |
| Nitrate (as Nitrogen) (ppm) | .58 | 2025 | 10 | 10 | No |  | Runoff from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits |  | (5 ppm through 10ppm) Nitrate in drinking water at levels above 10 ppm is a health risk for infants of less than six months of age. High nitrate levels in drinking water can cause blue baby syndrome. Nitrate levels may rise quickly for short periods of time because of rainfall or agricultural activity. If you are caring for an infant, you should ask for advice from your health care provider. (Above 10 ppm) Infants below the age of six months who drink water containing nitrate in excess of the MCL could become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blue baby syndrome. |
| Volatile Organic Contaminants |  |  |  |  |  |  |  |  |  |
| Contaminant (Units) | Level Detected* | Date | MCL | MCLG | Violation YES/NO |  | Likely Source of Contamination |  | Health Effects of Contaminant |
| Haloacetic Acids (HAA) (ppb) | 6.2 | 7/2025 | 60 | N/A | No |  | By-product of drinking water disinfection |  | Some people who drink water containing haloacetic acids in excess of the MCL over many years may have an increased risk of getting cancer. |
| Total Trihalomethanes (TTHM) (Bromodichloro-methane Bromoform Dibromochloro-methane Chloroform) (ppb) | 6.4 | 7/2025 | 80 | N/A | No |  | By-product of drinking water chlorination |  | Some people who drink water containing trihalomethanes in excess of the MCL over many years may experience problems with their liver, kidneys, or central nervous systems, and may have an increased risk of getting cancer. |

|  |  | PER- AND POLYFLUOROALKYL SUBSTANCES (PFAS) CONTAMINANTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contaminant (Units) |  |  | Level Detected* | Date | MCL | MCLG | Violation YES/NO | Likely Source of Contamination | Health Effects of Contaminant |
| Perfluorohexane sulfonic acid (PFHxS) (ppt) |  |  | .96 | 2025 | 18 | 0 | No | Discharge from industrial processes, wastewater treatment, residuals from firefighting foam, runoff/leachate from landfills and septic systems. | Some people who drink water containing perfluorohexane sulfonic acid (PFHxS) in excess of the MCL over many years could experience problems with their liver, endocrine system, or immune system, or may experience increased cholesterol levels. It may also lower a woman’s chance of getting pregnant. |
| Perfluorooctane sulfonic acid (PFOS) (ppt) |  |  | 1.11 | 2025 | 15 | 0 | No | Discharge from industrial processes, wastewater treatment, residuals from firefighting foam, runoff/leachate from landfills and septic systems. | Some people who drink water containing perfluorooctane sulfonic acid (PFOS) in excess of the MCL over many years could experience problems with their liver, endocrine system, or immune system, may experience increased cholesterol levels, and may have an increased risk of getting certain types of cancer. It may also lower a woman’s chance of getting pregnant. |
| Perfluorooctanoic acid (PFOA) (ppt) |  |  | 2.08 | 2025 | 12 | 0 | No | Discharge from industrial processes, wastewater treatment, residuals from firefighting foam, runoff/leachate from landfills and septic systems. | Some people who drink water containing perfluorooctanoic acid (PFOA) in excess of the MCL over many years could experience problems with their liver, endocrine system, or immune system, may experience increased cholesterol levels, and may have an increased risk of getting certain types of cancer. It may also lower a woman’s chance of getting pregnant. |

|  |  |  |  |  |  | SECONDARY CONTAMINANTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Secondary MCLs (SMCL) | Level Detected, Average & Range | Date | Treatment technique (if any) | SMCL |  | 50 % AGQS (Ambient groundwater quality standard) | AGQS (Ambient groundwater quality standard) | Specific contaminant criteria and reason for monitoring |
| Chloride (ppm) | 74.5 | 2025 | N/A | 250 |  | N/A | N/A | Wastewater, road salt, water softeners, corrosion |
| Manganese (ppm) | .007 | 2025 | N/A | 0.05 |  | 0.15 | 0.3 | Geological |
| pH | 6.8-8.4 | 2025 | N/A | 6.5-8.5 (Normal Range) |  | N/A | N/A | Precipitation and geology |
| Sodium (ppm) | 54.5 | 2025 | N/A | 100-250 |  | N/A | N/A | We are required to regularly sample for sodium |
| Sulfate (ppm) | 21 | 2025 | N/A | 250 |  | 250 | 500 | Naturally occurring |
| Zinc (ppm) | .0036 | 2025 | N/A | 5 |  | N/A | N/A | Galvanized pipes |